WEEK 6 FINAL PROJECT REPORT

IoT System Optimization and Final Project Synthesis

Optimized Smart Environmental Monitoring System

IoT Internship – Week 6
Simulation-Based Technical Project

Student Name: ______________________________
Institution: _________________________________
Date: ______________________________________

# 1. Abstract

This project presents a simulation-based optimization of a smart environmental monitoring IoT system.
The original design consists of 20 sensor nodes that periodically measure temperature and humidity and publish readings
through an MQTT-based gateway. The baseline configuration sends every sample, creating unnecessary network traffic,
higher processing overhead and increased energy use. The optimized configuration introduces event-driven transmission,
a periodic heartbeat mechanism, a smaller application payload and retry-aware communication. In the controlled
100-second simulation, transmissions decreased from 2000 to 701, average latency
decreased from 161.00 ms to 98.74 ms, and total simulated energy decreased
from 8701.56 mJ to 2036.30 mJ. The results show that application-level
optimization can improve IoT performance without requiring a complete redesign of the system.

# 2. Introduction

IoT systems combine sensing, communication, processing and application services. In a multi-node monitoring
system, sending every sensor reading can create unnecessary traffic when environmental values change slowly. As the
number of devices increases, this behavior can increase congestion, delay, retransmissions and energy consumption.
Therefore, the Week 6 objective is to review an existing IoT architecture, identify performance bottlenecks and apply
practical optimization techniques in a repeatable simulated environment.

The selected scenario is a smart environmental monitoring system for a building or small industrial facility.
Each sensor node measures temperature and humidity and communicates with a gateway. The gateway forwards data to an
MQTT broker and monitoring application. The project focuses on measurable improvements in latency, traffic volume,
reliability and energy-related communication cost.

# 3. Problem Statement

The baseline system transmits a message at every sampling interval, even when sensor values have not changed
enough to justify a new application-level update. This produces redundant traffic and consumes additional node and
network resources. The system can also experience packet loss, requiring retransmission and increasing end-to-end
latency. The problem is to improve performance while retaining regular monitoring and node-health visibility.

# 4. Original IoT System Design

## 4.1 Architecture

The baseline architecture contains four logical layers: sensing, connectivity, gateway/broker and application.
Sensor nodes periodically collect temperature and humidity values. Each reading is encapsulated into an application
message and sent to the gateway. The gateway forwards the data through an MQTT broker to a monitoring dashboard or
data service.

| Layer | Component | Function |
| --- | --- | --- |
| Sensing | 20 sensor nodes | Temperature and humidity acquisition |
| Connectivity | Wireless link | Transfers sensor messages to gateway |
| Edge/Gateway | IoT gateway | Receives, validates and forwards data |
| Messaging | MQTT broker | Publish/subscribe message delivery |
| Application | Monitoring service | Stores and visualizes measurements |

## 4.2 Baseline Data Flow

Sensor → wireless network → gateway → MQTT broker → monitoring application. The baseline node sends every sample regardless of the magnitude of change.

# 5. Performance Bottlenecks

Redundant periodic transmissions increase network load.

Every-message processing increases gateway and broker workload.

Packet loss can cause retransmissions and additional latency.

Large application payloads increase the number of transmitted bytes.

Continuous transmission consumes more energy than event-driven reporting.

Without a heartbeat, an aggressively event-driven design could hide node failures; therefore, reliability must be preserved during optimization.

# 6. Key Performance Metrics

| Metric | Purpose | Target |
| --- | --- | --- |
| Average latency | Measures typical communication delay | Reduce |
| P95 latency | Captures high-delay cases | Reduce |
| Transmission count | Measures redundant network activity | Reduce |
| Success rate | Measures delivery reliability | Maintain/increase |
| Retries | Measures recovery overhead | Reduce |
| Total bytes | Measures network traffic | Reduce |
| Simulated energy | Approximates communication cost | Reduce |

# 7. Optimization Strategy

## 7.1 Event-Driven Reporting

The optimized nodes transmit when the temperature changes by at least 0.8 °C or humidity changes by at least
2 percentage points since the last transmitted reading. This avoids sending values that are effectively unchanged.

## 7.2 Periodic Heartbeat

A heartbeat is transmitted every 10 sampling intervals. This provides a basic liveness signal even when
environmental values remain stable. The mechanism balances traffic reduction with operational visibility.

## 7.3 Smaller Payload

The optimized simulation represents a smaller application payload of 38 bytes instead of 54 bytes in the
baseline. In a real implementation, this could be achieved by compact serialization, removal of redundant fields,
binary encoding, or carefully selected MQTT payload structures.

## 7.4 Retry-Aware Reliability

The model includes packet loss and limited retransmission. The optimized configuration uses a lower modeled
packet-loss probability and lower processing overhead. The purpose is to represent the effect of a better-managed
communication path, while still allowing failure cases to occur.

## 7.5 Resource Allocation

The gateway is treated as an aggregation point so the application does not have to process every redundant
measurement independently. In a physical system, buffering, queue prioritization and connection management could be
used to further control resource utilization.

# 8. Optimized System Architecture

The optimized flow is: Sensor → event filter → wireless link → gateway aggregation → MQTT broker → monitoring
application. The event filter is placed close to the sensor so that unnecessary messages are suppressed before they
consume network capacity. A periodic heartbeat ensures that inactivity is not confused with device failure.

| Component | Baseline | Optimized |
| --- | --- | --- |
| Reporting | Every sample | Change threshold + heartbeat |
| Payload | 54 bytes | 38 bytes |
| Modeled loss probability | 4.5% | 1.2% |
| Processing overhead | Higher | Lower |
| Reliability signal | Continuous data only | Data + heartbeat |

# 9. Simulation Methodology

The simulation models 20 sensor nodes over 100 seconds. Each node produces 100
samples, giving 2000 sample opportunities. Temperature and humidity values are generated using
sinusoidal environmental trends plus random measurement noise. The baseline transmits every sample. The optimized
system applies the change thresholds and heartbeat rule. Latency, packet loss, retries, payload size and a simplified
energy cost are generated using controlled random distributions. A fixed random seed is used so the run is reproducible.

The simulation is intentionally an educational model rather than a physical-network measurement. Its purpose
is to demonstrate a defensible before-and-after optimization workflow and to provide logs that can be inspected,
recalculated and reproduced.

# 10. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Number of nodes | 20 |
| Samples per node | 100 |
| Simulation duration | 100 seconds |
| Sampling interval | 1 second |
| Temperature change threshold | 0.8 °C |
| Humidity change threshold | 2.0 percentage points |
| Heartbeat interval | 10 samples |
| Baseline payload | 54 bytes |
| Optimized payload | 38 bytes |
| Random seed | 42 |

# 11. Benchmark Results

| Metric | Baseline | Optimized | Change |
| --- | --- | --- | --- |
| Transmissions | 2000 | 701 | 65.0% lower |
| Average latency | 161.00 ms | 98.74 ms | 38.7% lower |
| P95 latency | 203.24 ms | 119.50 ms | 41.2% lower |
| Success rate | 98.95% | 99.71% | +0.76 pp |
| Retries | 53 | 5 | 90.6% lower |
| Total bytes | 108000 | 26638 | 75.3% lower |
| Total simulated energy | 8701.56 mJ | 2036.30 mJ | 76.6% lower |

# 12. Comparative Analysis

The largest improvement is in communication volume. The optimized system reduces transmissions by
65.0%, which directly reduces the number of messages competing
for wireless and gateway resources. Total payload bytes fall by 75.3%.
This reduction is important because radio transmission is typically one of the more expensive operations for
battery-powered IoT nodes.

Latency also improves in the simulation. Average latency falls from 161.00 ms to
98.74 ms, while P95 latency falls from 203.24 ms to
119.50 ms. This represents lower modeled processing and communication overhead and fewer
retry-related delays.

Reliability improves from 98.95% to 99.71% in the modeled
environment. The retry count also decreases, which means fewer resources are spent recovering from communication
failures. These values should be interpreted as simulation outcomes rather than guarantees for physical hardware.

# 13. Robustness Evaluation

The heartbeat mechanism prevents long periods of silence from being interpreted as guaranteed normal operation.

Retry logic allows selected packet-loss events to be recovered.

Threshold-based reporting reduces traffic while retaining significant environmental changes.

A fixed simulation seed allows the benchmark to be reproduced.

The architecture remains compatible with MQTT-style publish/subscribe communication.

Additional physical testing would be required to validate RF interference, battery behavior and real network congestion.

# 14. Troubleshooting and Design Decisions

| Observed issue | Root cause | Solution |
| --- | --- | --- |
| High traffic volume | Every sample transmitted | Event-driven reporting |
| Latency spikes | Loss/retry and processing overhead | Lower modeled loss and retry overhead |
| Energy overhead | Frequent transmissions | Suppress redundant messages |
| Risk of missing node failure | Pure event-driven reporting | Periodic heartbeat |
| Large messages | Unnecessary payload fields | Compact payload design |

# 15. Limitations

The project uses a simulated communication and energy model. It does not emulate a specific radio standard,
antenna, physical channel, broker implementation or real battery chemistry. The latency and packet-loss values are
therefore representative assumptions for an academic benchmark. A future hardware validation should use actual ESP32
or equivalent nodes, a real MQTT broker, measured RSSI, packet delivery ratio, CPU utilization and battery-current
measurements.

# 16. Future Enhancements

Implement the design on ESP32 sensor nodes.

Compare MQTT QoS levels under controlled packet-loss conditions.

Add edge aggregation and local anomaly detection.

Measure real battery current and sleep/wake duty cycle.

Test Wi-Fi versus a low-power mesh protocol.

Add dashboard visualization and long-term time-series storage.

Perform stress testing with 50, 100 and 500 simulated nodes.

# 17. Conclusion

The Week 6 project successfully integrates IoT architecture, communication, optimization and performance
analysis into one consolidated scenario. The optimized design reduces redundant transmissions, lowers modeled latency,
reduces communication energy cost and improves reliability while retaining node-health visibility through heartbeats.
In the simulation, total transmissions decreased from 2000 to 701 and total
simulated energy decreased by 76.6%. The project demonstrates
that practical optimization does not always require replacing the complete IoT architecture; targeted improvements at
the sensing, messaging and gateway layers can produce meaningful performance gains.

# 18. Project Files

The accompanying submission package contains the Python simulation source, generated CSV logs, comparison
results and this Word report. The same files can be placed into a GitHub repository for evaluation.

| File | Purpose |
| --- | --- |
| simulation/iot_optimization_simulation.py | Reproducible simulation source |
| logs/baseline_log.csv | Baseline node-level simulation log |
| logs/optimized_log.csv | Optimized node-level simulation log |
| results/comparison.csv | Before/after benchmark summary |
| README.md | GitHub project documentation |
| docs/Week6_IoT_Optimization_Report.docx | Final technical report |

# 19. References

OASIS. MQTT Version 3.1.1 specification. MQTT publish/subscribe messaging concepts.

Python Software Foundation. Python 3 documentation.

Internship Week 1–5 learning material and previous IoT tasks, as applicable.

# Appendix A – Reproduction Instructions

1. Install Python 3.9 or later. 2. Open the repository folder in a terminal.
3. Run `python simulation/iot_optimization_simulation.py`. 4. Inspect the generated CSV logs in `logs/`.
5. Compare the resulting metrics with `results/comparison.csv`. 6. Upload the repository to GitHub and submit the
repository URL together with this Word report.